Supplementary Material

Supplementary Figures and Tables

Supplementary Tables

| object | equipment | Equipment parameters | Equipment capacity |
| --- | --- | --- | --- |
| Distributed energy station 1 | photovoltaic | Shape factor: 0.79 Scale coefficient: 1.87 Rated light intensity: 700 | Total PV capacity ：4MW |
|  | transformer | T1: 35/11kV T2: 35/11kV | T1: 5MVA T1: 6MVA |
|  | GB | Energy conversion efficiency: 0.9 | 6MW |
|  | CP1、CP2 | Energy conversion efficiency: 0.8 | Rated power: 0.5MW |
|  | C1、C2 | Boost ratio: 1.2 | 6MW |
|  | CHP unit | CCHP，min: 0.1 CCHP，max: 1.2 | Capacitance: 4MW Heat capacity: 4MW |
| Distributed energy station 2 | Fan | Scale coefficient: 2.15 Shape factor: 15 Cut in wind speed：4m/s Rated wind speed: 15m/s Cut out wind speed: 25m/s | Total fan capacity : 4MW |
|  | transformer | T3: 35/11kV T4: 35/11kV | T3: 6MVA T4: 6MVA |
|  | compressor | C1 Boost ratio: 1.20 C2 Boost ratio: 1.20 | 6MW |
| Critical pipeline | Power pipeline I | （L1、L2、L10、L7） Voltage level: 10kV | Capacity: 6.31MVA |
|  | Power pipeline II | （L3、L4）Voltage level: 10kV | Capacity: 7.5MVA |
|  | Heat supply network pipeline | Water supply temperature: 100℃ Return water temperature: 49-50℃ | Capacity: 89.81m3/h |
|  | Natural gas pipeline | Subhigh pressure natural gas pipeline network Pressure: 0.8~1.6MPa | 0.334MMCFD2 |