Appendix for

# Distribution, sources and health risk assessment of polycyclic aromatic hydrocarbons in urban soils under different landform conditions of Taiyuan, China

Liwen Liu1,Xiangling Chen1,Yonghong Duan1,*, Zhangwei Wu1,Lishuai Xu1

1. College of Resources and Environment, Shanxi Agricultural University, Taigu, Shanxi 030801, China;

* Corresponding author: Prof. Yonghong Duan, Tel.: 0086-354- 6288322. E-mail: yhduanpku@126.com

This material includes:

Table S1. Statistics for PAHs concentrations (ng g-1) in surface soil samples in the Plain

Table S2. Statistics for PAHs concentrations (ng g-1) in the surface soil samples in Mountain

Table S3. PAHs concentrations (ng g-1) in surface soil samples from different areas

Figure S1. Wind rose based on meteorological data from 2000 to 2018 of Taiyuan city

Figure S2. Fla/(Fla+Pyr) and BaA/(BaA+Chr) of PAHs in different landform of Taiyuan city

Table S4. Diagnostic ratios of PAHs in different landform of Taiyuan city

Table S5. Factor loadings for the principal factors extracted for the polycyclic aromatic hydrocarbon concentrations in soil

Table S6. Parameters used to calculate the incremental lifetime cancer risks

Table S7. TEF values of individual PAHs for calculation of BaPeq

Table S8. Incremental lifetime cancer risks (ILCRs) for soil from the Taiyuan City

References

Table S1. Statistics for PAHs concentrations (ng g-1) in surface soil samples in the Plain

| Component | Detection rate(%) | Mean | Median | Min | Max | STDa |
| --- | --- | --- | --- | --- | --- | --- |
| NAP | 100 | 21.3 | 12.4 | 0.6 | 149.8 | 25.2 |
| ANY | 100 | 9.0 | 6.3 | 1.4 | 49.1 | 9.3 |
| ACE | 100 | 4.0 | 2.0 | 0.1 | 29.8 | 5.3 |
| FLO | 100 | 15.3 | 10.0 | 0.0 | 82.0 | 15.2 |
| PHE | 100 | 120.6 | 80.8 | 9.3 | 499.8 | 107.8 |
| ANT | 100 | 147.2 | 105.2 | 9.6 | 542.2 | 132.7 |
| RET | 100 | 49.1 | 40.2 | 13.7 | 149.6 | 33.0 |
| FLA | 100 | 188.4 | 116.4 | 14.0 | 793.0 | 192.8 |
| PYR | 100 | 117.6 | 68.0 | 8.5 | 499.5 | 124.4 |
| BaA | 100 | 107.9 | 58.3 | 7.1 | 596.1 | 144.6 |
| CHR | 100 | 94.3 | 60.2 | 7.4 | 381.2 | 95.9 |
| BbF | 100 | 149.1 | 90.0 | 16.1 | 705.0 | 157.5 |
| BkF | 100 | 41.5 | 25.8 | 3.4 | 195.0 | 47.1 |
| BaP | 100 | 83.5 | 44.5 | 7.6 | 425.1 | 103.4 |
| DahA | 100 | 11.7 | 7.4 | 0.0 | 64.4 | 12.7 |
| PER | 100 | 15.2 | 8.1 | 1.5 | 78.9 | 18.6 |
| BeP | 100 | 101.2 | 63.2 | 9.9 | 443.9 | 106.0 |
| CPP | 100 | 27.5 | 14.3 | 0.0 | 232.7 | 42.2 |
| IcdP | 100 | 56.8 | 29.9 | 0.0 | 335.1 | 72.8 |
| BghiP | 100 | 71.9 | 43.7 | 0.3 | 322.8 | 79.5 |
| COR | 100 | 11.7 | 6.5 | 0.0 | 57.2 | 12.2 |
| Σ7CarPAHs | 100 | 544.8 | 336.2 | 49.6 | 2657.9 | 615.0 |
| Σ16PAHs | 100 | 1240.0 | 907.4 | 100.1 | 5448.3 | 1277.3 |
| Σ21PAHs | 100 | 1444.7 | 1072.8 | 133.2 | 6410.6 | 1451.3 |

a Standard deviation.

b Not detected.

Table S2. Statistics for PAHs concentrations (ng g-1) in the surface soil samples in Mountain

| Component | Detection rate(%) | Mean | Median | Min | Max | STDa |
| --- | --- | --- | --- | --- | --- | --- |
| NAP | 100 | 11.9 | 7.9 | 0.0 | 103.7 | 21.9 |
| ANY | 100 | 3.6 | 2.9 | 0.3 | 25.9 | 5.4 |
| ACE | 100 | 1.4 | 0.7 | 0.0 | 11.1 | 2.5 |
| FLO | 100 | 10.6 | 5.6 | 0.0 | 80.0 | 17.8 |
| PHE | 100 | 66.6 | 46.1 | 6.8 | 404.8 | 93.0 |
| ANT | 100 | 82.3 | 53.6 | 7.0 | 535.5 | 124.1 |
| RET | 100 | 26.8 | 22.4 | 7.7 | 53.1 | 14.0 |
| FLA | 100 | 69.9 | 48.2 | 7.0 | 346.5 | 79.9 |
| PYR | 100 | 41.7 | 27.0 | 4.4 | 209.7 | 49.1 |
| BaA | 100 | 33.9 | 15.2 | 1.9 | 187.0 | 45.9 |
| CHR | 100 | 37.4 | 23.7 | 3.6 | 154.9 | 43.2 |
| BbF | 100 | 53.8 | 40.3 | 3.1 | 190.6 | 50.0 |
| BkF | 100 | 13.1 | 8.6 | 1.3 | 57.0 | 13.9 |
| BaP | 100 | 24.1 | 13.8 | 2.0 | 106.4 | 25.9 |
| DahA | 100 | 4.3 | 3.0 | 0.4 | 14.8 | 3.7 |
| PER | 100 | 4.3 | 2.7 | 0.4 | 21.6 | 5.0 |
| BeP | 100 | 36.7 | 21.1 | 2.8 | 134.0 | 37.6 |
| CPP | 100 | 11.8 | 5.0 | 0.0 | 68.2 | 16.0 |
| IcdP | 100 | 21.6 | 16.9 | 0.0 | 96.9 | 22.4 |
| BghiP | 100 | 24.2 | 17.1 | 2.9 | 104.0 | 24.8 |
| COR | 100 | 5.5 | 4.5 | 0.9 | 17.1 | 4.2 |
| Σ7CarPAHs | 100 | 188.0 | 111.1 | 12.8 | 803.2 | 198.4 |
| Σ16PAHs | 100 | 500.3 | 338.2 | 50.6 | 2065.8 | 566.9 |
| Σ21PAHs | 100 | 585.5 | 428.8 | 66.5 | 2250.2 | 617.6 |

a Standard deviation.

b Not detected.

Table S3. PAHs concentrations (ng g-1) in surface soil samples from different areas

| Location | Type | No. of PAHs | Min | Max | Mean | Year | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Plain |  |  |  |  |  |  |  |
| Shanghai, China | Urban and rural | 16 | 18.8 | 6320 | 807 | 2011 | (Wang et al., 2015) |
| Beijing and Tianjin，China | Urban and rural | 16 | 31.6 | 1475 | 336 | 2007 | (Wang et al., 2010) |
| Harbin, China | Urban | 16 | 17 | 3260 | 508 | 2006 | (Ma et al., 2009) |
| Lisbon, Portugal | Urban | 16 | 6.3 | 22670 | 1544 | - | (Cachada et al., 2012) |
| Kurukshetra, India | Urban | 16 | 16.1 | 2538 | 632 | 2012 | (Kumar et al., 2013) |
| Ulsan, Korea | Urban | 16 | 65 | 12000 | 960 | 2010 | (Kwon and Choi, 2014) |
| Nanjing,China | urban | 16 | 41.19 | 7016.65 | 979.59 | - | (Yang et al., 2017) |
| Xi'an,China | urban | 16 | 390.6 | 10652.8 | 2052.6 | - | (Wang et al., 2018) |
| Chengdu,China | urban | 16 | 12.52 | 75,431.47 | 3233.92 | 2006 | (Xing et al., 2011) |
| Orlando,USA | urban | 16 | 43 | 30426 | 3227 | 2016 | (Liu et al., 2019) |
| Mountain |  |  |  |  |  |  |  |
| Qinling, China | Mountain edges | 16 | 25.11 | 10129.4 | 748.03 | 2017-2019 | (Chen et al., 2022) |
| Changbai，China | Mountain | 16 | 38.5 | 190 | 84.7 | 2009 | (Zhao et al., 2015) |
| Wangtiane，china | Mountain | 16 | 362 | 666 | 491 | 2016 | (Wang et al., 2019) |
| Remote woodland，UK | Mountain | 15 | 42 | 4850 | 580 | 1998 | (Nam et al., 2008) |
| Eastern Himalayas，India | Mountain | 16 | 15.3 | 4762 | 458 | 2014 | (Devi et al., 2016) |

Figure S1 Wind rose based on meteorological data from 2000 to 2018 of Taiyuan city

| (a) | (b) |
| --- | --- |

Figure S2 Fla/(Fla+Pyr) and BaA/(BaA+Chr) of PAHs in different landform of Taiyuan city

Table S4. Diagnostic ratios of PAHs in different landform of Taiyuan city

| Diagnostic ratios | Fla/(Fla+Pyr) | BaA/(BaA+Chr) |  | Fla/(Fla+Pyr) | BaA/(BaA+Chr) |
| --- | --- | --- | --- | --- | --- |
| Plain |  |  | Mountain |  |  |
| min | 0.60 | 0.30 | min | 0.55 | 0.27 |
| max | 0.66 | 0.61 | max | 0.66 | 0.65 |
| mean | 0.63 | 0.45 | mean | 0.62 | 0.47 |
| STD | 0.02 | 0.08 | STD | 0.02 | 0.08 |

Table S5. Factor loadings for the principal factors extracted for the polycyclic aromatic hydrocarbon concentrations in soil

|  | In the Plain |  | In Mountain |  |  |
| --- | --- | --- | --- | --- | --- |
|  | PC1 | PC2 | PC1 | PC2 | PC3 |
| NAP | 0.66 | 0.27 | 0.16 | 0.96 | -0.15 |
| ANY | 0.63 | 0.65 | 0.20 | 0.97 | -0.01 |
| ACE | 0.47 | 0.79 | 0.97 | 0.07 | -0.12 |
| FLO | 0.46 | 0.74 | 0.36 | 0.92 | -0.09 |
| PHE | 0.82 | 0.51 | 0.49 | 0.86 | -0.09 |
| ANT | 0.88 | 0.38 | 0.49 | 0.85 | -0.13 |
| RET | 0.34 | 0.61 | -0.06 | -0.06 | 0.98 |
| FLA | 0.91 | 0.39 | 0.98 | 0.17 | -0.05 |
| PYR | 0.92 | 0.36 | 0.97 | 0.20 | -0.08 |
| BaA | 0.91 | 0.38 | 0.96 | 0.20 | -0.14 |
| CHR | 0.91 | 0.37 | 0.73 | 0.67 | -0.08 |
| BbF | 0.85 | 0.49 | 0.84 | 0.50 | 0.08 |
| BkF | 0.93 | 0.35 | 0.96 | 0.26 | -0.07 |
| BaP | 0.91 | 0.38 | 0.96 | 0.25 | -0.05 |
| DahA | 0.80 | 0.58 | 0.84 | 0.51 | 0.05 |
| PER | 0.90 | 0.38 | 0.99 | 0.12 | -0.06 |
| BeP | 0.90 | 0.42 | 0.75 | 0.66 | -0.02 |
| CPP | 0.11 | 0.93 | -0.10 | 0.90 | 0.30 |
| IcdP | 0.65 | 0.59 | 0.97 | 0.21 | 0.06 |
| BghiP | 0.90 | 0.38 | 0.91 | 0.41 | -0.05 |
| COR | 0.62 | 0.65 | 0.83 | 0.47 | 0.15 |

Extraction method: principal component analysis.

Rotation method: varimax with Kaiser norm.

Table S6. Parameters used to calculate the incremental lifetime cancer risks

| Parameter | Unit | Child | Adult | Reference |
| --- | --- | --- | --- | --- |
| BW | kg | 15 | 61.5 | (Jia et al., 2021) |
| EF | day/year | 180 | 180 | (Jia et al., 2021) |
| ED | year | 6 | 24 | (Jia et al., 2021) |
| IRinhalation | m3/day | 10 | 20 | (Jia et al., 2021) |
| IRingestion | mg/day | 200 | 100 | (Jia et al., 2021) |
| SA | cm2/day | 2800 | 5700 | (Jia et al., 2021) |
| AF | mg/cm2 | 0.2 | 0.07 | (Jia et al., 2021) |
| ABS |  | 0.13 | 0.13 | (Jia et al., 2021) |
| AT | day | 25550 | 25550 | (Jia et al., 2021) |
| PEF | m3/kg | 1.36×10109 | 1.36×109 | (Jia et al., 2021) |

Table S7. TEF values of individual PAHs for calculation of BaPeq

| Abbrev | Full Name | TEF | Abbrev | Full Name | TEF |  |
| --- | --- | --- | --- | --- | --- | --- |
| NAP | naphthalene | 0.001a | BkF | benzo(k)fluoranthene |  | 0.100a |
| ACY | acenaphthylene | 0.001a | BaP | benzo(a)pyrene |  | 1.000a |
| ACE | acenaphthene | 0.001a | IcdP | indeno(l,2,3-cd)pyrene |  | 0.100a |
| FLO | fluorene | 0.001a | DahA | dibenz(a,h)anthracene |  | 1.000a |
| PHE | phenanthrene | 0.001a | BghiP | benzo(g,h,i)perylene |  | 0.010a |
| ANT | anthracene | 0.010a | PER | perylene |  | 0.001b |
| FLA | fluoranthene | 0.001a | BeP | benzo(e)pyrene |  | 0.010b |
| PYR | pyrene | 0.001a | COR | Coronene |  | 0.001b |
| BaA | benzo(a)anthracene | 0.100a | CPP | cyclopenta[c,d]pyrene |  | 0.100b |
| CHR | chrysene | 0.010a | RET | retene |  | 0.040 c |
| BbF | benzo(b)fluoranthene | 0.100a |  |  |  |  |

a (Nisbet et al., 1992)

b (Malcolm and Dobson, 1994)

c (Billiard et al., 2004)

Table S8. Incremental lifetime cancer risks (ILCRs) for soil from the Taiyuan City

| Exposure pathway | Child |  |  |  | Adult |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ILCRingestion | ILCRdermal | ILCRinhalation | Cancer risk | ILCRingestion | ILCRdermal | ILCRinhalation | Cancer risk |
| In plain |  |  |  |  |  |  |  |  |
| min | 3.4×10-8 | 4.2×10-8 | 6.6×10-13 | 7.6×10-8 | 2.6×10-8 | 4.7×10-8 | 2.1×10-12 | 7.3×10-8 |
| max | 1.8×10-6 | 2.2×10-6 | 3.4×10-11 | 4.0×10-6 | 1.4×10-6 | 2.4×10-6 | 1.1×10-10 | 3.8×10-6 |
| mean | 3.4×10-7 | 4.3×10-7 | 6.7×10-12 | 7.7×10-7 | 2.7×10-7 | 4.8×10-7 | 2.1×10-11 | 7.5×10-7 |
| median | 2.0×10-7 | 2.4×10-7 | 3.8×10-12 | 4.4×10-7 | 1.5×10-7 | 2.7×10-7 | 1.2×10-11 | 4.2×10-7 |
| In mountain |  |  |  |  |  |  |  |  |
| min | 9.8×10-9 | 1.2×10-8 | 1.9×10-13 | 2.2×10-8 | 7.7×10-9 | 1.4×10-8 | 5.9×10-13 | 2.1×10-8 |
| max | 4.5×10-7 | 5.6×10-7 | 8.7×10-12 | 1.0×10-6 | 3.5×10-7 | 6.2×10-7 | 2.7×10-11 | 9.8×10-7 |
| mean | 1.1×10-7 | 1.4×10-7 | 2.1×10-12 | 2.5×10-7 | 8.6×10-8 | 1.5×10-7 | 6.7×10-12 | 2.4×10-7 |
| median | 6.9×10-8 | 8.6×10-8 | 1.3×10-12 | 1.6×10-7 | 5.4×10-8 | 9.6×10-8 | 4.2×10-12 | 1.5×10-7 |

# References

Billiard S, Bols N, Hodson P (2004) In vitro and in vivo comparisons of fish-specific CYP1A induction relative potency factors for selected polycyclic aromatic hydrocarbons. Ecotoxicol Environ Saf 59:292-299.

Cachada A, Pato P, Rocha-Santos T, da Silva EF, Duarte A (2012) Levels, sources and potential human health risks of organic pollutants in urban soils. Sci Total Environ 430:184-192.

Chen Z, Tian Z, Liu X, Sun W (2022) The potential risks and exposure of Qinling giant pandas to polycyclic aromatic hydrocarbon (PAH) pollution. Environ Pollut 292:118294. doi:https://doi.org/10.1016/j.envpol.2021.118294

Devi NL, Yadav IC, Shihua Q, Dan Y, Zhang G, Raha P (2016) Environmental carcinogenic polycyclic aromatic hydrocarbons in soil from Himalayas, India: Implications for spatial distribution, sources apportionment and risk assessment. Chemosphere 144:493-502.

Jia T, Guo W, Xing Y, Lei R, Wu X, Sun S, He Y, Liu W (2021) Spatial distributions and sources of PAHs in soil in chemical industry parks in the Yangtze River Delta, China. Environ Pollut 283:117121.

Kwon H-O, Choi S-D (2014) Polycyclic aromatic hydrocarbons (PAHs) in soils from a multi-industrial city, South Korea. Sci Total Environ 470:1494-1501.

Liu Y, Gao P, Su J, da Silva EB, de Oliveira LM, Townsend T, Xiang P, Ma LQ (2019) PAHs in urban soils of two Florida cities: background concentrations, distribution, and sources. Chemosphere 214:220-227.

Ma W-L, Li Y-F, Sun D-Z, Qi H (2009) Polycyclic aromatic hydrocarbons and polychlorinated biphenyls in topsoils of Harbin, China. Arch Environ Contam Toxicol 57:670-678.

Malcolm H, Dobson S (1994) The Calculation of an Environmental Assessment Level (EAL) for Atmospheric PAHs Using Relative Potencies; Department of the Environment, London

Nisbet IC, Lagoy PK (1992) Toxic equivalency factors (TEFs) for polycyclic aromatic hydrocarbons (PAHs). Regul Toxicol Pharmacol 16:290-300.

Shen G, Tao S, Wei S, Zhang Y, Simonich SLM (2012a) Retene Emission from Residential Solid Fuels in China and Evaluation of Retene as a Unique Marker for Soft Wood Combustion. Environ Sci Technol 46:4666-4672.

Shen G, Tao S, Wei S, Zhang Y, Wang R, Wang B, Li W, Shen H, Huang Y, Chen Y (2012b) Emissions of parent, nitro, and oxygenated polycyclic aromatic hydrocarbons from residential wood combustion in rural China. Environ Sci Technol 46:8123-8130.

Wang L, Zhang P, Wang L, Zhang W, Shi X, Lu X, Li X, Li X (2018) Polycyclic aromatic hydrocarbons in urban soil in the semi-arid City of Xi’an, Northwest China: Composition, distribution, sources, and relationships with soil properties. Arch Environ Contam Toxicol 75:351-366.

Wang X-T, Chen L, Wang X-K, Lei B-L, Sun Y-F, Zhou J, Wu M-H (2015) Occurrence, sources and health risk assessment of polycyclic aromatic hydrocarbons in urban (Pudong) and suburban soils from Shanghai in China. Chemosphere 119:1224-1232.

Wang Z, Liu S, Zhang T (2019) Characteristics of polycyclic aromatic hydrocarbons (PAHs) in soil horizon from high-altitude mountains in Northeastern China. Chemosphere 225:93-103.

Xing X, Qi S, Zhang J, Wu C, Zhang Y, Yang D, Odhiambo JO (2011) Spatial distribution and source diagnosis of polycyclic aromatic hydrocarbons in soils from Chengdu Economic Region, Sichuan Province, western China. J Geochem Explor 110:146-154.

Yang J, Yu F, Yu Y, Zhang J, Wang R, Srinivasulu M, Vasenev V (2017) Characterization, source apportionment, and risk assessment of polycyclic aromatic hydrocarbons in urban soil of Nanjing, China. J Soils Sediments 17:1116-1125.

Zhao X, Kim S-K, Zhu W, Kannan N, Li D (2015) Long-range atmospheric transport and the distribution of polycyclic aromatic hydrocarbons in Changbai Mountain. Chemosphere 119:289-294.