# Appendix A: Interpretation indices for RMD-induced reservoir geohazards in the HFB

Table A.1 Interpretation indices for RMD-induced reservoir geohazards in the HFB

| Indices | Qualitative description |  |
| --- | --- | --- |
|  | Carbonate HFB | Clastic HFB |
| Level 1: Interpretation of factors triggering instability |  |  |
| RMD | Indirectly interpreted by vegetation development, such as no vegetation or scattered vegetation |  |
|  | Directly interpreted by the enhancement of macroscopic defects of rock masses, such as the development of solution fissures, grooves, and pores, the widening, deepening, and extension of discontinuities, the fracturing, spallings and fallings of rock blocks, the formation of piles of rubble accumulating at the bank slope toe |  |
| Level 2: Interpretation of factors controlling failure mode |  |  |
| Lithology | Limestone; Dolomitic limestone; Dolomite | Sandstone; Shale, Interbedded sandstone-mudstone, Sandstone interbedded with muddy shale |
| Rock mass structure | Intact blocky structure; Layered structure; Fractured structure; Loosened structure | Intact blocky structure; Layered structure; Fractured structure; Loosened structure; Soft-hard interbedded structure. |
| RMD’s type | Dissolution-suffosion; Crack development-extension; Mechanical erosion | Loosening-spalling; Erosion-abrasion; Collapsing-block falling near structural planes; Weak strip interlayer denudation |
| Geomorphological characteristics | Cliffs and steep slopes appear alternately in the middle-low mountains or deep-cut V-shaped valleys, The overall inclination of the steep slopes is 40  50° |  |
| Bank slope structure | Consequent bedding slopes; Oblique bedding slopes; Reverse slopes; Gentle stratified slopes; Tower-shaped slopes |  |
| Feature of structural plane | Large or controlling structural planes formed by several large or medium-sized original discontinuities or RMD-induced unloading fissures |  |
| Level 3: Interpretation of visible signs reflecting evolution state |  |  |
| Boundary characteristics | Main scarp at the rear edge of potential geohazards has formed, several small gullies or minor scarps with microrelief changes emerge near the boundary, and rock masses near the front edge are severely deteriorated. Rock mass are highly fractured due to extrusion, forming piles of rubble |  |

# Appendix B: Development characteristics and evolution mode of potential geohazard in the study area

Table B.1 Development characteristics and evolution mode of potential geohazard in the study area

| Geohazard ID | Field site ID | Area (m2) | Volume (×104 m3) | Rock type | Potential evolution mode | Existing geohazard site |
| --- | --- | --- | --- | --- | --- | --- |
| XL1 | xl30 | 4908.09 | 2.45 | Clastic | Bedding sliding | Hengliangzi unstable slope |
| XL2 | xl29 | 4833.11 | 2.42 | Clastic | Reverse toppling | Hengliangzi unstable slope |
| XL3 | xl28 | 4119.89 | 2.06 | Clastic | Reverse toppling | / |
| XL4 | xl27 | 11234.65 | 5.62 | Clastic | Reverse toppling | / |
| XL5 | xl26 | 9295.77 | 4.65 | Clastic | Reverse toppling | / |
| XL6 | xl25 | 3231.19 | 1.62 | Clastic | Soft-hard interbedded rock collapse | / |
| XL7 | xl24 | 4576.37 | 2.29 | Clastic | Reverse toppling | / |
| XL8 | xl23 | 3514.45 | 1.76 | Clastic | Bedding sliding | / |
| XL9 | xl22 | 1685.26 | 0.84 | Clastic | Bedding sliding | / |
| XL10 | xl21 | 11084.65 | 5.54 | Clastic | Reverse toppling | / |
| XL11 | xl20 | 1070.16 | 0.54 | Clastic | Reverse toppling | / |
| XL12 | xl19 | 2483.92 | 1.24 | Clastic | Soft-hard interbedded rock collapse | / |
| XL13 | xl18 | 1856.22 | 0.93 | Clastic | Soft-hard interbedded rock collapse | / |
| XL14 | xl17 | 5107.56 | 2.55 | Clastic | Soft-hard interbedded rock collapse | / |
| XL15 | xl16 | 10198.18 | 5.10 | Clastic | Reverse toppling | Ganjuchang landslide in Badou village |
| XL16 | xl15 | 5253.44 | 2.63 | Clastic | Reverse toppling | / |
| XL17 | xl14 | 2617.65 | 1.31 | Clastic | Reverse toppling | / |
| XL18 | xl13 | 2349.23 | 1.17 | Clastic | Soft-hard interbedded rock collapse | / |
| XL19 | xl12 | 3791.08 | 1.90 | Clastic | Reverse toppling | / |
| XL20 | xl11 | 1949.16 | 0.97 | Clastic | Soft-hard interbedded rock collapse | / |
| XL21 | xl10 | 4272.79 | 2.14 | Clastic | Soft-hard interbedded rock collapse | Ganjuchang landslide in Shimen village |
| XL22 | xl9 | 3154.76 | 1.58 | Clastic | Soft-hard interbedded rock collapse | Ganjuchang unstable slope in Changcao village |
| XL23 | xl8 | 7274.39 | 3.64 | Clastic | Reverse toppling | Ganjuchang unstable slope in Changcao village |
| XL24 | xl7 | 9722.55 | 4.86 | Clastic | Reverse toppling | / |
| XL25 | xl6 | 4068.85 | 2.03 | Clastic | Soft-hard interbedded rock collapse | / |
| XL26 | xl5 | 1796.64 | 0.90 | Clastic | Soft-hard interbedded rock collapse | Zhafangju landslide |
| XL27 | xl4 | 1730.19 | 0.87 | Clastic | Reverse toppling | Daozuokeng landslide |
| XL28 | xl3 | 11221.15 | 5.61 | Clastic | Soft-hard interbedded rock collapse | / |
| XL29 | xl2 | 17638.39 | 8.82 | Clastic | Soft-hard interbedded rock collapse | / |
| XL30 | xl1 | 7427.77 | 3.71 | Clastic | Soft-hard interbedded rock collapse | Yangjiaping landslide |
| XL31 | xl46 | 5395.47 | 2.70 | Clastic | Reverse toppling | Xiaoyaogutuo landslide |
| XL32 | xl45 | 7437.37 | 3.72 | Clastic | Bedding sliding | / |
| XL33 | xl44 | 18286.34 | 9.14 | Clastic | Soft-hard interbedded rock collapse | / |
| XL36 | xl41 | 14060.60 | 7.03 | Clastic | Bedding sliding | / |
| XL37 | xl40 | 18037.41 | 9.02 | Clastic | Bedding sliding | / |
| XL38 | xl39 | 37719.40 | 18.86 | Clastic | Bedding sliding | / |
| XL41 | xl36 | 11432.93 | 5.72 | Clastic | Bedding sliding | Lianyushan unstable slope |
| XL42 | xl35 | 19680.04 | 9.84 | Clastic | Soft-hard interbedded rock collapse | / |
| XL43 | xl34 | 4360.30 | 2.18 | Clastic | Soft-hard interbedded rock collapse | / |
| XL44 | xl33 | 8660.18 | 4.33 | Clastic | Reverse toppling | Tanjiacao unstable slope |
| XL45 | xl32 | 872.70 | 0.44 | Clastic | Reverse toppling | Tanjiacao unstable slope |
| XL46 | xl31 | 3927.26 | 1.96 | Clastic | Reverse toppling | Tanjiacao unstable slope |
| XL47 | xl68 | 3858.48 | 1.93 | Carbonate | Toe-emptying induced toppling | / |
| XL48 | xl67 | 3772.02 | 1.89 | Carbonate | Base-crushing | / |
| XL49 | xl66 | 3999.73 | 2.00 | Carbonate | Base-crushing | / |
| XL50 | xl65 | 1317.98 | 0.66 | Carbonate | Toe-emptying induced toppling | / |
| XL51 | xl64 | 2541.02 | 1.27 | Carbonate | Bedding sliding | / |
| XL52 | xl63 | 3508.19 | 1.75 | Carbonate | Toe-emptying induced toppling | / |
| XL53 | xl62 | 1587.02 | 0.79 | Carbonate | Bedding sliding | / |
| XL54 | xl61 | 4759.93 | 2.38 | Carbonate | Base-crushing | / |
| XL55 | xl60 | 3280.85 | 1.64 | Carbonate | Bedding sliding | / |
| XL56 | xl59 | 4564.12 | 2.28 | Carbonate | Bedding sliding | / |
| XL57 | xl58 | 7834.22 | 3.92 | Carbonate | Bedding sliding | Unstable bank slopes from Longju to shangxiaoren village |
| XL58 | xl57 | 11062.66 | 5.53 | Carbonate | Reverse toppling | Unstable bank slopes from Longju to shangxiaoren village |
| XL59 | xl56 | 2207.25 | 1.10 | Carbonate | Toe-emptying induced toppling | / |
| XL60 | xl55 | 1314.10 | 0.66 | Carbonate | Base-crushing | / |
| XL61 | xl54 | 2802.21 | 1.40 | Carbonate | Bedding sliding | / |
| XL62 | xl53 | 3557.12 | 1.78 | Clastic | Soft-hard interbedded rock collapse | / |
| XL63 | xl52 | 1530.04 | 0.77 | Carbonate | Bedding sliding | North Guangjiawan collapse |
| XL64 | xl51 | 5266.92 | 2.63 | Carbonate | Base-crushing | / |
| XL65 | xl50 | 1727.18 | 0.86 | Carbonate | Toe-emptying induced toppling | / |
| XL66 | xl49 | 2249.36 | 1.12 | Carbonate | Toe-emptying induced toppling | / |
| XL67 | xl48 | 7546.31 | 3.77 | Carbonate | Base-crushing | / |
| XL68 | xl47 | 26460.47 | 13.23 | Carbonate | Base-crushing | / |
| XR1 | xr1 | 10634.90 | 5.32 | Carbonate | Bedding sliding | / |
| XR2 | xr2 | 5101.88 | 2.55 | Carbonate | Bedding sliding | / |
| XR3 | xr3 | 1330.00 | 0.66 | Carbonate | Bedding sliding | Wangjiaju landslide |
| XR4 | xr4 | 2966.61 | 1.48 | Clastic | Soft-hard interbedded rock collapse | / |
| XR5 | xr5 | 7002.84 | 3.50 | Clastic | Bedding sliding | Cangping landslide |
| XR6 | xr6 | 1860.08 | 0.93 | Clastic | Bedding sliding | / |
| XR7 | xr7 | 1128.72 | 0.56 | Clastic | Bedding sliding | / |
| XR8 | xr8 | 9506.09 | 4.75 | Clastic | Reverse toppling | / |
| XR10 | xr10 | 4419.11 | 2.21 | Clastic | Soft-hard interbedded rock collapse | / |
| XR11 | xr11 | 563.42 | 0.28 | Clastic | Bedding sliding | East Taiziwan landslide |
| XR12 | xr12 | 45829.80 | 22.91 | Clastic | Apparent-dip wedge sliding | Yangjiatuo landslide |
| XR13 | xr13 | 10648.27 | 5.32 | Clastic | Bedding sliding | Yangjiatuo landslide |
| XR14 | xr14 | 22453.16 | 11.23 | Clastic | Bedding sliding | Tongzigou landslide |
| XR15 | xr15 | 12536.97 | 6.27 | Clastic | Bedding sliding | Group 1# landslide in Shenshuicao village |
| XR16 | xr16 | 7275.97 | 3.64 | Clastic | Soft-hard interbedded rock collapse | / |
| XR17 | xr17 | 31034.14 | 15.52 | Clastic | Bedding sliding | Zaozishubao landslide |
| XR18 | xr18 | 20490.26 | 10.25 | Clastic | Bedding sliding | Harbor landslide in Shaxi town |
| XR19 | xr19 | 7021.44 | 3.51 | Clastic | Bedding sliding | / |
| XR20 | xr20 | 6143.39 | 3.07 | Clastic | Soft-hard interbedded rock collapse | / |
| XR21 | xr21 | 1576.08 | 0.79 | Clastic | Bedding sliding | / |
| XR22 | xr22 | 25821.00 | 12.91 | Clastic | Bedding sliding | / |
| XR23 | xr23 | 5544.81 | 2.77 | Clastic | Soft-hard interbedded rock collapse | / |
| XR24 | xr24 | 2111.04 | 1.06 | Clastic | Soft-hard interbedded rock collapse | / |
| XR25 | xr25 | 2020.42 | 1.01 | Clastic | Soft-hard interbedded rock collapse | / |
| XR26 | xr26 | 5100.47 | 2.55 | Clastic | Bedding sliding | / |
| XR27 | xr27 | 12891.96 | 6.45 | Clastic | Bedding sliding | / |
| XR28 | xr28 | 5227.69 | 2.61 | Clastic | Bedding sliding | / |
| XR29 | xr29 | 3737.38 | 1.87 | Clastic | Bedding sliding | / |
| XR30 | xr30 | 3562.17 | 1.78 | Clastic | Bedding sliding | / |
| XR31 | xr31 | 8938.76 | 4.47 | Clastic | Bedding sliding | / |
| XR32 | xr32 | 10114.21 | 5.06 | Clastic | Soft-hard interbedded rock collapse | / |
| XR33 | xr33 | 39441.92 | 19.72 | Clastic | Soft-hard interbedded rock collapse | Shangshimen landslide |
| XR34 | xr34 | 10671.49 | 5.34 | Clastic | Apparent-dip wedge sliding | / |
| XR35 | xr35 | 45187.50 | 22.59 | Clastic | Soft-hard interbedded rock collapse | / |
| XR36 | xr36 | 13070.25 | 6.54 | Clastic | Soft-hard interbedded rock collapse | Kangshapo landslide |
| XR37 | xr37 | 26607.46 | 13.30 | Clastic | Soft-hard interbedded rock collapse | / |
| XR38 | xr38 | 2304.83 | 1.15 | Carbonate | Base-crushing | / |
| XR39 | xr39 | 2235.84 | 1.12 | Carbonate | Base-crushing | / |
| XR40 | xr40 | 3301.21 | 1.65 | Carbonate | Base-crushing | / |
| XR41 | xr41 | 8951.49 | 4.48 | Carbonate | Bedding sliding | / |
| XR42 | xr42 | 29042.62 | 14.52 | Clastic | Bedding sliding | Xiaoxintan unstable slope |
| XR43 | xr43 | 26977.83 | 13.49 | Clastic | Soft-hard interbedded rock collapse | Guangfuyuan landslide |
| XR44 | xr44 | 14392.13 | 7.20 | Clastic | Bedding sliding | Group 1# landslide in Xiling village |
| XR45 | xr45 | 4661.97 | 2.33 | Clastic | Soft-hard interbedded rock collapse | Fengzhuping landslide |
| XR46 | xr46 | 2522.19 | 1.26 | Clastic | Soft-hard interbedded rock collapse | Fengzhuping landslide |
| XR47 | xr47 | 6443.77 | 3.22 | Clastic | Soft-hard interbedded rock collapse | Fengzhuping landslide |
| XR48 | xr48 | 12088.98 | 6.04 | Clastic | Reverse toppling | Xiangjiabao landslide |
| XR50 | xr50 | 7330.73 | 3.67 | Clastic | Soft-hard interbedded rock collapse | / |
| XR51 | xr51 | 3448.15 | 1.72 | Carbonate | Base-crushing | / |
| XR52 | xr52 | 20654.17 | 10.33 | Carbonate | Base-crushing | / |
| XR53 | xr53 | 55604.35 | 27.80 | Carbonate | Bedding sliding | / |
| XR54 | xr54 | 86406.32 | 43.20 | Carbonate | Toe-emptying induced toppling | Shangjijiawan unstable slope |