Application of the optimized decoupling methodology for the construction of a skeletal primary reference fuel (PRF) mechanism focusing on engine-relevant conditions

Yachao Chang, Ming Jia *, Yaopeng Li, Maozhao Xie

School of Energy and Power Engineering, Dalian University of Technology, Dalian, P.R. China

Appendix A. Comparison of the latest reduced PRF mechanisms with the present mechanism

Table S1 Comparison of the size of the latest reduced mechanisms

| Author | Species number | Reaction number |  |
| --- | --- | --- | --- |
| Ra and Reitz (2008) | 41 | 130 |  |
| Tsurushima (2009) | 33 | 38 |  |
| Luong et al. (2013) | 171 | 861 |  |
| Wang et al. (2013) | 73 | 296 |  |
| Present mechanism | 49 | 163 |  |

Table S1 compares the size of the latest reduced PRF mechanisms with that of the present mechanism. The smallest PRF mechanism is obtained by Tsurushima (2009) based on the work of Tanaka et al. (2003) focusing on ignition delay time in shock tubes and species evolution in HCCI engines. Ra and Reitz (2008) constructed a reduced PRF mechanism using a combination of chemical lumping, graphical reaction flow analysis and elimination methods. By reducing the detailed mechanism form Mehl et al. (Mehl et al.) with the directed relation graph with error propagation (DRGEP) approach, Wang et al. (2013) built a reduced PRF mechanism consisting of 73 species and 296 reactions. Consistently, using the DRG-based methods, isomer lumping and timescale analysis, Luong et al. (2013) reduced a detailed PRF mechanism to a 171-species reduced mechanism. Because most of the previous reduced mechanisms for PRF are constructed based on the overall reduction of the detailed mechanisms focusing on a specific operating condition, further validation of the mechanisms listed in Table S1 in a wide range of temperature, pressure, and equivalence ratio are presented as follows.

Figure S1 Comparison of the measured (symbols) (Fieweger et al., 1997) and predicted (lines) ignition delay times for various PRFs at p=4.0 MPa and φ=1.0

Figure S1 compares the predicted ignition delay time using the latest reduced PRF mechanisms listed in Table S1 and the present mechanism with the measurements. As can be seen, the predicted ignition delay times using the Ra and Reitz mechanism, Tsurushima mechanism, and Wang et al. mechanism show similar behavior for the fuel with octane number (ON) above 60, and are all in good agreement with the experimental data for PRF90 and PRF100. However, the ignition delay time is slightly underestimated in the NTC and high-temperature regions for PRF60 and PRF80 by the above three mechanisms. For PRF0, the Ra and Reitz mechanism well predicts the ignition delay time over the whole temperature range, while the Wang et al. mechanism slightly over-predicts the ignition delay and Tsurushima mechanism under-predicts the ignition delay time in the low temperature region. The Luong et al. mechanism shows the longest predicted ignition delay time in the low-to-intermediate temperature, especially considerable overprediction of the ignition delay time is observed in the temperature range between 850 and 1000 K for PRF90 and PRF100. For the present mechanism, the ignition delay time is  satisfactorily reproduced in the test operating conditions, as shown in Figure S1.

Figure S2 Comparison of the measured (symbols) (van Lipzig et al., 2011) and predicted (lines) laminar flame speed at p=0.1 MPa and (a) Tu=298 K and (b) Tu=338 K

Comparison of the performance on the prediction of the laminar flame speed using various mechanisms listed in Table S1 is presented in Figure S2. As can be seen, the Ra and Reitz mechanism overpredicts the laminar flame speed for all the test cases, while the Tsurushima mechanism considerably underestimates the measurements. For the Wang et al. mechanism, the laminar flame speed is slightly under-predicted over the whole equivalence ratio range tested herein. The predicted laminar flame speed using the Luong et al. mechanism and the present mechanism agree with the experimental data reasonably well under the tested operating conditions, as shown in Figure S2.

Figure S3 Comparison of the measured (symbols) (Dagaut et al., 1994) and predicted (lines) major species concentrations in a JSR for 0.1% PRF90/air mixture at p=1.0 MPa, τ=1 s, and φ=1.0

Figure S3 shows comparison of the behavior of the predicted major species concentrations using the mechanisms listed in Table S1. It can be seen that the low temperature reactivity is overpredicted by the Tsurushima mechanism, Luong et al. mechanism, and Wang et al. mechanism. At high temperatures, the CO2 concentration is overpredicted, whereas the CO concentration is underestimated by all the tested mechanisms.

Overall, the predictions from the present mechanism are in best agreement with the measurements on the ignition delay time, laminar flame speed, and major species concentrations over a wide range of temperature, pressure, and equivalence ratio for various PRFs.

References:

Dagaut, P., Reuillon, M., and Cathonnet, M. (1994). High pressure oxidation of liquid fuels from low to high temperature. 2. Mixtures of n-heptane and iso-octane. Combust. Sci. Technol. 103, 315-336. doi: 10.1080/00102209408907701.

Fieweger, K., Blumenthal, R., and Adomeit, G. (1997). Self-ignition of s.I. Engine model fuels: A shock tube investigation at high pressure. Combust. Flame 109, 599-619. doi: 10.1016/S0010-2180(97)00049-7.

Luong, M.B., Luo, Z., Lu, T., Chung, S.H., and Yoo, C.S. (2013). Direct numerical simulations of the ignition of lean primary reference fuel/air mixtures with temperature inhomogeneities. Combust. Flame 160, 2038-2047. doi: 10.1016/j.combustflame.2013.04.012.

Mehl, M., Pitz, W.J., Westbrook, C.K., and Curran, H.J. (2011). Kinetic modeling of gasoline surrogate components and mixtures under engine conditions. Proc. Combust. Inst. 33, 193-200. doi: 10.1016/j.proci.2010.05.027.

Ra, Y., and Reitz, R.D. (2008). A reduced chemical kinetic model for ic engine combustion simulations with primary reference fuels. Combust. Flame 155, 713-738. doi: 10.1016/j.combustflame.2008.05.002.

Tanaka, S., Ayala, F., and Keck, J.C. (2003). A reduced chenmical kinetic model for hcci combustion of primary reference fuels in a rapid compression machine. Combust. Flame 133, 467-481.

Tsurushima, T. (2009). A new skeletal PRF kinetic model for HCCI combustion. Proc. Combust. Inst. 32, 2835-2841. doi: 10.1016/j.proci.2008.06.018.

Van Lipzig, J.P.J., Nilsson, E.J.K., De Goey, L.P.H., and Konnov, A.A. (2011). Laminar burning velocities of n-heptane, iso-octane, ethanol and their binary and tertiary mixtures. Fuel 90, 2773-2781. doi: 10.1016/j.fuel.2011.04.029.

Wang, H., Yao, M., and Reitz, R.D. (2013). Development of a reduced primary reference fuel mechanism for internal combustion engine combustion simulations. Energy Fuels 27, 7843-7853. doi: 10.1021/ef401992e.