General LC-MS metabolite profiling - Polaris

Metabolite profiling was performed using a QToF 6520 mass spectrometer (Agilent Technologies, Palo Alto, USA) coupled to a 1200 series Rapid Resolution HPLC system. 5 µl of sample extract was loaded onto a Polaris 3 C18-A 1.8 µm, 2.0 x 250 mm reverse phase analytical column (Agilent Technologies, Palo Alto, USA). For detection using positive ion mode, mobile phase A comprised 0.1% formic acid in water and mobile phase B was 95% acetonitrile with 0.1% formic acid in water. For negative ion mode, mobile phase A comprised 5mM ammonium formate in water and mobile phase B was 95% acetonitrile.

The following gradient was used: 0 min – 0% B; 10 min – 0% B; 30 min – 100% B; 40 min – 100% B; 40.5 min – 0% B; 10 min post time. The flow rate was 0.2 ml min-1 and the column temperature was held at 35 °C for the duration. The source conditions for electrospray ionisation were as follows: gas temperature was 325 °C with a drying gas flow rate of 9 l min-1 and a nebuliser pressure of 35 psig. The capillary voltage was 3.5 kV in both positive and negative ion mode. The fragmentor voltage was 115 V and skimmer 70 V. Scanning was performed using the autoMS/MS function with a survey scan of 5 scans sec-1, and MS/MS scan rate of 4 scans sec-1. A sloped collision energy of 3.5 V/100 Da with an offset of 5 V was applied for untargeted collision induced dissociation.