Supplementary Material

Meta-atoms Library

We set up a meta-atoms library composed of multiple germanium nanopillars. When light travels through a nanostructure, a portion of the light is confined inside the nanostructure, while the rest leaks into the surrounding material. This is similar to what occurs in a waveguide. Owing to this similarity, the optical properties of nanostructures can be better understood by treating them as miniature truncated waveguides. As illustrated in Supplementary Figure 1, three meta-atom archetypes are employed in our library: one nanofin corresponds to a rectangular miniature waveguide, two nanofins correspond to slotted miniature waveguides, and three nanofins correspond to multislotted miniature waveguides.  Supplementary Figures 1 A to C show the conversion efficiency and phase spectra for the three selected meta-atoms. The phases of the three meta-atoms were linear with respect to the frequency within the operating bandwidth, and the group delays of the three meta-atoms were 0.82 ps, 1.13 ps, and 0.67 ps, respectively. Supplementary Figure 1D shows the top and side views of the normalized magnetic energy density in a periodic array for the three selected nanostructures. Owing to the waveguide-like effect, the light was observed to mostly remain within the nanostructures. This indicates that the group delay design for the elements was accurate even when arranged in a square lattice as a metasurface array; thus, the light coupling effects between adjacent pairs of elements can be ignored.

Supplementary Figure 1. Simulation results for three selected meta-atoms. (A) Phase and conversion efficiency spectrum for one nanofin, geometry parameters in the inset picture: L1 = 4.4 μm, W1 = 2.2 μm; (B) Phase and conversion efficiency spectrum for two nanofins, geometry parameters in the inset picture: L1 = 4.3 μm, W1 = 1.3 μm, g1 = 0.5 μm, L2 = 1.9 μm, W2 = 1.9 μm; (C) Phase and conversion efficiency spectrum for three nanofins, geometry parameters in the inset picture: L1 = 1.3 μm, W1 = 1 μm, g1 = 0.5 μm, L2 = 2.1 μm, W2 = 1 μm, L3 = 1.1 μm, W3 = 1 μm; (D) top and side views of normalized H field intensities for the three meta-atoms.

Metasurface generating optical vortices with  and

To confirm the universal nature of our method, we show the metasurfaces to generate optical vortices of different orders. Supplementary Figure 2 and 3 show the simulation results of metasurface generating optical vortices with . Supplementary Figure 4 and 5 show the simulation results of metasurface generating optical vortices with .

Supplementary Figure 2. Intensity and phase distribution at the focal plane for metasurface generating OAM with.

Supplementary Figure 3. Intensity distribution on XOZ plane for metasurface generating OAM with.

Supplementary Figure 4. Intensity and phase distribution at the focal plane for metasurface generating OAM with.

Supplementary Figure 5. Intensity distribution on XOZ plane for metasurface generating OAM with.

Transmittance of the meta-atoms

Supplementary Figure 6. Transmittance of the selected meta-atoms shown in Figure 2C.