Supplementary material

TABLE S1 Calibration of parameters in water quality (including phytoplankton) model.

| Parameters | Description | Reference range | Value |
| --- | --- | --- | --- |
| ANDC (The amount of NO3 consumed dissolved carbon) | Mass of NO3 consumed per unit of dissolved organic carbon oxidation |  | 0.933 |
| rNitM (Rate of nitrification maximum) | Maximum nitrification rate (g N m3/day) | 0.001～1.3 | 0.1 |
| KHNitDO (Key coefficient of half-saturation nitrification dissolved oxygen) | Dissolved oxygen half-saturation coefficient of nitrification |  | 0.5 |
| KHNitN (Key coefficient of half-saturation nitrification nitrogen) | Semi-saturation coefficient of ammonia nitrogen for nitrification |  | 0.05 |
| TNit (Temperature of nitrification) | Reference temperature of nitrification (°C) |  | 30 |
| KLP (Key rate of lytic phosphorus) | Hydrolysis rates of soluble organic phosphorus particles (/d) | 0.01～0.63 | 0.08 |
| KDP (Key rate of dissolved phosphorus) | Mineralization rate of dissolved organic phosphorus (/d) | 0.01～0.63 | 0.08 |
| KRC (Key rate of refractory carbon) | Dissolution rate of insoluble particulate organic carbon (/d) | 0.001 | 0.001 |
| KLC (Key rate of lytic carbon) | Dissolution rate of soluble particulate organic carbon (/d) | 0.01～0.63 | 0.06 |
| KDC (Key rate of dissolved carbon) | Decomposition rate of dissolved organic carbon (/d) | 0.01～0.63 | 0.1 |
| KRP (Key rate of refractory phosphorus) | Hydrolysis rate of insoluble particulate organophosphorus (/d) | 0.001 | 0.001 |
| KRN (Key rate of refractory nitrogen) | Hydrolysis rate of insoluble particulate organic nitrogen (/d) | 0.001 | 0.001 |
| KLN (Key rate of lytic nitrogen) | Hydrolysis rate of soluble particulate organic nitrogen (/d) | 0.01～0.63 | 0.05 |
| KDN (Key rate of dissolved nitrogen) | Mineralization rate of dissolved organic nitrogen (/d) | 0.01～0.63 | 0.05 |
| WSrp (Whereabout speed of refractory particulate) | Sedimentation rate of insoluble particulate organic matter (m/d) | 0.02～9.0 | 0.2 |
| WSlp (Whereabout ppeed of lytic particulate) | Sedimentation rate of soluble particulate organic matter (m/d) | 0.02～9.0 | 0.2 |
| PMc (Production maximum of cyanobacteria) | Maximum growth rate of cyanobacteria (/d) | 0.2～9.0 | 1.6 |
| TMRc1 (Temperature minimum regenerate of cyanobacteria) | Lower limit of optimum temperature for cyanobacteria growth (°C) |  | 28 |
| KHNx (Key coefficient of half-saturation nitrogen) | Nitrogen half-saturation for algae (mg/L) | 0.006～4.32 | 0.012 |
| FCLP (Flux of carbon labile particle) | Carbon partition coefficient of predatory algae: active particulate organic carbon |  | 0.4 |
| FPLP (Flux of phosphorus labile particle) | Phosphorus partition coefficient of predatory algae: active particulate organic phosphorus |  | 0.3 |
| FNLP (Flux of nitrogen labile particle) | Nitrogen partition coefficient of predatory algae: active particulate organic nitrogen |  | 0.3 |
| BMRc (Basal metabolism rate for cyanobacteria) | Cyanobacteria basal respiratory rate (/d) | 0.01～0.92 | 0.05 |
| PRRc (Predation rate on refractory cyanobacteria) | Predation Rate on algae (/d) | 0.03～0.3 | 0.07 |
| WSc (Whereabout speed of cyanobacteria) | Setting velocity for algae (m/d) | 0.001～13.2 | 0.05 |
| KHPx (Key coefficient of half-saturation phosphorus) | Phosphorus half-saturation for algae (mg/L) | 0.001～1.52 | 0.001 |
| KEb (Key extinction coefficient of background) | Background extinction coefficient (/m) | 0.10～0.45 | 0.10 |
| KECHL (Key extinction coefficient of chlorophyll) | Extinction coefficient of suspended chlorophyll (1/m per ug/L) | 0.002～0.02 | 0.012 |

FIGURE S1 Comparison of simulated and measured water levels at four hydrological stations.

FIGURE S2 Comparison of simulated and measured TN concentrations of Lake Poyang in 2015.

FIGURE S3 Comparison of simulated and measured TP concentrations of Lake Poyang in 2015.

FIGURE S4 Comparison of simulated and measured Chl a concentrations of Lake Poyang in 2015.